## Appendix/Technical Reference

This appendix is the engineering / maintenance reference. It intentionally contains the deeper technical details that are not placed in the client-facing docs. If you are an operator/client and you are only trying to use the system, you usually do not need this appendix.

## A1. Bill of Materials

### Vest Unit (Transmitter)

| Component | Quantity | Notes |
| --- | --- | --- |
| ESP32 DevKit V1 | 1 | Main controller |
| LoRa SX1278 (Ra-02) | 1 | Wireless transmission |
| GPS Module GY-NEO6MV2 | 1 | Location tracking |
| Rain Sensor (plate + module) | 1 set | Water detection |
| Lithium Battery (18650) | 3 | Power source |
| Battery Holder (3-slot) | 1 | Holds batteries |
| DC-DC Step Down Regulator | 2 | One for 5V rail, one for 3.3V rail |
| Switch | 1 | Power control |
| Capacitor 100µF | 1 | For LoRa stability |
| Capacitor 10µF | 1 | For GPS stability |
| Wires | --- | Connections |
| Perfboard | 1 | Mounting |
| Connectors | --- | Connections |

### Control Tower (Receiver)

| Component | Quantity | Notes |
| --- | --- | --- |
| ESP32 DevKit V1 | 1 | Main controller |
| LoRa SX1278 (Ra-02) | 1 | Receive data |
| Power Supply (USB) | 1 | Stable power |
| Jumper Wires | --- | Connections |
| Perfboard | 1 | Mounting |

## A2. Circuit Schematic

## A3. Pin Connections / Wiring Tables

### Vest Unit Wiring (Power Chain)

| Component | Pin/Label | Connected To | Notes |
| --- | --- | --- | --- |
| Battery Holder (3x Lithium) | + | Switch input | Main power |
| Battery Holder | – | Common GND | Ground |
| Switch | Output | DC-DC (5V rail) IN+ and DC-DC (3.3V rail) IN+ | Power control |
| DC-DC (5V rail) | IN+ | Switch output | Input power |
| DC-DC (5V rail) | IN– | GND | Common ground |
| DC-DC (5V rail) | OUT+ (5V) | ESP32 VIN | 5V rail |
| DC-DC (5V rail) | OUT– | GND | Common ground |
| DC-DC (3.3V rail) | IN+ | Switch output | Input power |
| DC-DC (3.3V rail) | IN– | GND | Common ground |
| DC-DC (3.3V rail) | OUT+ (3.3V) | LoRa VCC, GPS VCC and rain sensor VCC | 3.3V rail |
| DC-DC (3.3V rail) | OUT– | GND | Common ground |

### Vest Unit Wiring (ESP32 Pin Map)

| ESP32 | Connected To | Notes |
| --- | --- | --- |
| VIN | 5V (from regulator) | Power input |
| GND | Common GND | Shared ground |
| 3V3 | Not used for powering modules | Use dedicated DC‑DC 3.3V rail instead |
| GPIO16 (RX2) | GPS TX | UART |
| GPIO17 (TX2) | GPS RX (optional) | Can leave unconnected |
| GPIO18 | LoRa SCK | SPI |
| GPIO19 | LoRa MISO | SPI |
| GPIO23 | LoRa MOSI | SPI |
| GPIO5 | LoRa NSS (CS) | SPI select |
| GPIO14 | LoRa RST | Reset |
| GPIO26 | LoRa DIO0 | Interrupt |
| GPIO34 | Rain Sensor AO | Analog input |

### LoRa (SX1278 / Ra-02) Wiring

| LoRa Pin | Connected To | Notes |
| --- | --- | --- |
| VCC | DC-DC (3.3V rail) OUT+ | 3.3V only (do not power from ESP32 3V3) |
| GND | GND |  |
| SCK | GPIO18 |  |
| MISO | GPIO19 |  |
| MOSI | GPIO23 |  |
| NSS | GPIO5 |  |
| RST | GPIO14 |  |
| DIO0 | GPIO26 |  |

### GPS (GY-NEO6MV2) Wiring

| GPS Pin | Connected To | Notes |
| --- | --- | --- |
| VCC | DC-DC (3.3V rail) OUT+ | 3.3V only (do not power from ESP32 3V3) |
| GND | GND |  |
| TX | GPIO16 | Data to ESP32 |
| RX | GPIO17 (optional) | Can leave open |

### Rain Sensor Module Wiring

| Rain Module | Connected To | Notes |
| --- | --- | --- |
| VCC | 5V rail or 3.3V rail | Prefer 3.3V rail for stable |
| GND | GND |  |
| AO | GPIO34 | Analog reading |
| DO | Not used | Optional only |

## A4. Capacitor Placement (Reference)

| Location | Capacitor | Connection |
| --- | --- | --- |
| LoRa | 100µF | 3.3V rail to GND (near module) |
| GPS | 10µF | 3.3V rail to GND (near module) |

## A5. Assembly Notes (Reference Only)

### Vest Unit Assembly (High level)

- Wire the power chain: Battery holder to Switch to DC‑DC (5V rail) and DC‑DC (3.3V rail).

- Connect GPS to ESP32 UART2.

- Connect LoRa to ESP32 SPI (power LoRa from the DC‑DC 3.3V rail, not ESP32 3V3).

- Connect Rain Sensor analog output to the ESP32 analog input.

- Add the recommended capacitors close to the LoRa and GPS modules.

### Control Tower Assembly (High level)

- Connect LoRa module to ESP32 (same pin map as vest LoRa wiring).

- Power ESP32 via USB or regulated 5V supply.

## A6. Optional Operator Quick Reference

### Glossary

- LoRa: Long-range, low-power radio link used between the vest and control tower.

- Control tower: The receiver unit that listens for LoRa packets and uploads them to the database.

- RTDB: Firebase Realtime Database where control tower uploads packets when available.

- GPS lock: GPS has a fresh valid position fix.

### Normal timing expectations

- Scheduled LoRa transmit: about once per minute (per vest).

- Event LoRa transmit: immediately when the sensor transitions from dry to wet.

A7. ESP32 Pin Out

A7. LoRa SX1278 Pin Out

A8.  GY-NEO6MV2 Pin Out

A9.  Rain Sensor Pin Out