Firmware & Software

This document summarizes what the shipped code does and how major pieces fit together. It is not a substitute for reading the sketches under `firmware/` or the Flutter sources under `android-app/vest_tracker/lib/`. For operator facing steps, see MobileApp.pdf and UserGuide.pdf. For pin maps and BOM, see Appendix.pdf.

FS1. Repository layout (reference)

| Path | Role |
| --- | --- |
| `firmware/TRANSMITTER_VEST/` | Vest ESP32: GPS, rain analog, LoRa TX |
| `firmware/RECEIVER_CONTROL_TOWER_V2/` | Receiver ESP32: LoRa RX, Wi‑Fi, Firebase RTDB upload, message queue, Malaysia `time_my` |
| `firmware/RECEIVER_CONTROL_TOWER/` | Earlier receiver variant (simpler, prefer V2 for production) |
| `firmware/*_testing_only/` | Bring-up sketches (GPS, LoRa, rain, Firebase isolation) |
| `android-app/vest_tracker/` | Flutter Vest Tracker Android app |
| `hardware/schematic/` | KiCad schematic (`vest_tracker_schematic.kicad_sch`) |

Security note: receiver `.ino` files contain placeholder Wi‑Fi and Firebase constants. Replace them with deployment-specific values before flashing. Do not commit real production secrets to a public repository.

FS2. Vest unit (transmitter)

Source: `firmware/TRANSMITTER_VEST/TRANSMITTER_VEST.ino`

Responsibilities

- Parse NEO-6M GPS NMEA on ESP32 UART2 (RX `GPIO16`, TX `GPIO17`, 9600 baud).

- Sample rain sensor analog on GPIO34 with debouncing.

- Transmit compact JSON over LoRa on a timer and on wet edges.

- Emit human-readable USB Serial JSON (10 s) for maintenance 115200 baud.

Compile-time identity: `VEST_ID` (default `1`) > maps to RTDB `device_01` at receiver.

GPS stage state machine

| `gs` | Constant | `gsn` string | Entry condition |
| --- | --- | --- | --- |
| `1` | `GPS_STAGE_WAITING_FOR_SIGNAL` | `waiting_for_signal` | No GPS bytes within 2000 ms (`GPS_NO_DATA_TIMEOUT_MS`) |
| `2` | `GPS_STAGE_AVAILABLE_NO_LOCK` | `no_satellite_lock` | Bytes present but no valid fix, or fix age ≥ 2000 ms |
| `3` | `GPS_STAGE_LOCKED` | `locked` | Valid lat/lon and fix age < 2000 ms |

`lastGoodFix` behavior: When stage 3 is reached, firmware stores lat, lon, alt, speed, satellites, fix timestamp. LoRa payloads prefer this stored snapshot even if live stage later drops to 1 or 2, operators may see stale coordinates with fresh wet status until a new outdoor lock.

LoRa transmit timing

| Trigger | Interval / rule |
| --- | --- |
| Scheduled TX | Every 60 s (`LORA_TX_INTERVAL_MS`) |
| Event TX | Immediately on debounced dry to wet edge |
| Rain sample skip | Skipped on 60 s uptime boundary and when scheduled LoRa TX is due (avoids ADC read during RF) |
| Serial JSON skip | Also suppressed on 60 s boundary tick |

No deep sleep, vest runs continuously with 1 ms loop delay. Boot delay 1000 ms. LoRa init failure to infinite hang (“LoRa INIT FAILED”).

Rain detection

| Constant | Value | Meaning |
| --- | --- | --- |
| `RAIN_ANALOG_WET_THRESHOLD` | `2500` | Wet when `analogRead(GPIO34) < 2500` |
| `RAIN_DEBOUNCE_MS` | `80` | Stable state must hold 80 ms before change |

- `wetEdgeToTrue`: set only on transition dry to wet (used to trigger immediate LoRa TX).

- `imp` (important): in current logic equals stable wet state (true whenever wet, not edge-only).

JSON formats

- USB Serial (diagnostics, 10 s, buffer 512 bytes)

  - Long field names: `gps_stage`, `gps_stage_name`, `latitude`, `longitude`, `altitude_m`, `speed_kmh`, `satellites`, `fix_age_ms`, `water_detection`, `important_event`, `uptime_ms`.

  - Lat/lon precision: 6 decimal places.

- LoRa (compact radio, buffer 240 bytes)

  - With stored fix:

{"id":1,"gs":3,"gsn":"locked","lat":14.59950,"lon":120.98420,"alt":15,"spd":0.0,"sat":7,"fix_ms":123456,"wet":false,"imp":false,"t":654321}

  - Without stored fix:

{"id":1,"gs":1,"gsn":"waiting_for_signal","loc":false,"wet":false,"imp":false,"t":654321}

| Field | Meaning |
| --- | --- |
| `id` | Vest numeric ID to RTDB path |
| `gs`, `gsn` | GPS stage number and name |
| `lat`, `lon`, `alt`, `spd`, `sat`, `fix_ms` | Present when including position snapshot |
| `loc` | When `false`, receiver treats location as absent |
| `wet` | Rain boolean |
| `imp` | Important flag (stable wet in current logic) |
| `t` | Transmitter uptime (ms) |

LoRa radio: 433 MHz (`433E6`), SPI pins SS=`5`, RST=`14`, DIO0=`2`, SCK/MISO/MOSI =`18`/`19`/`23`.

FS3. Control tower (receiver V2)

Source: `firmware/RECEIVER_CONTROL_TOWER_V2/RECEIVER_CONTROL_TOWER_V2.ino`

Responsibilities

- LoRa RX on same frequency/pinout as vest.

- Parse LoRa JSON (`parseVestJson`, `StaticJsonDocument<512>`).

- Enqueue messages in RAM, flush to Firebase when Wi‑Fi and Firebase ready.

- NTP sync to attach `time_my` (Malaysia UTC+8, ISO-8601 `YYYY-MM-DDTHH:MM:SS+08:00`).

- Resilience: exponential backoff, token-not-ready handling, hard recovery after repeated RTDB failures.

Message queue

| Constant | Value |
| --- | --- |
| `QUEUE_CAPACITY` | 30 messages |
| `MAX_FLUSH_PER_LOOP` | 4 RTDB writes per loop iteration |

Ring buffer: when full, oldest message is dropped before push. LoRa RX continues even when Wi‑Fi is down, extended outages can lose historical snapshots (RTDB always stores latest state per device, not full history).

Wi‑Fi / Firebase / RTDB resilience

Three `Backoff` instances (`wifiBackoff`, `firebaseBackoff`, `rtdbWriteBackoff`):

Initial delay 250 ms, max 30000 ms, exponential doubling.

Other timing:

| Constant | Value | Purpose |
| --- | --- | --- |
| Wi-Fi re-`begin` interval | 60000 ms | If disconnected too long |
| NTP timeout | 20000 ms | `pool.ntp.org`, `time.nist.gov`, UTC+8 |
| Firebase socket/response timeout | 20000 ms each |  |
| Firebase Wi-Fi reconnect timeout | 10000 ms |  |
| `POST_FIREBASE_BEGIN_QUIET_MS` | 7000 ms | No RTDB writes immediately after `Firebase.begin()` |
| Token-not-ready wait | 5000 ms | Does not count as hard RTDB failure |
| `MAX_CONSECUTIVE_RTDB_FAILURES_BEFORE_HARD_RECOVERY` | 6 | Triggers full recovery |
| `HARD_RECOVERY_COOLDOWN_MS` | 60000 ms | Between hard recoveries |

- Hard recovery: reset Firebase session, disconnect Wi‑Fi, clear NTP synced flag, reset all backoffs, clears stuck SSL/TCP state.

- RTDB write API: single atomic `Firebase.RTDB.updateNode` per message (not per-field sets).

LoRa > RTDB field mapping

Path rule: `/vest/device_XX` where `XX` is zero-padded to two digits when `id < 10` (`id=1` is `device_01`, `id=10` is `device_10`).

| LoRa / source | RTDB key | Notes |
| --- | --- | --- |
| `id` | path segment | See rule above |
| `gs` | `gs` | default 0 if missing |
| `gsn` | `gsn` | max 31 chars |
| `wet` | `wet` | bool |
| `imp` | `imp` | bool |
| `t` | `t` | uint32 uptime ms |
| `lat`, `lon`, `alt`, `spd`, `sat`, `fix_ms` | same keys | Only if lat+lon present and `loc` not explicitly `false` |
| receiver NTP | `time_my` | Omitted if NTP never synced |

Example RTDB documents (after V2 upload)

Locked with location:

{

"gs": 3,

"gsn": "locked",

"lat": 14.5995,

"lon": 120.9842,

"alt": 15,

"spd": 0.0,

"sat": 7,

"fix_ms": 123456,

"wet": false,

"imp": false,

"t": 654321,

"time_my": "2026-05-30T14:32:01+08:00"

}

No fix:

{

"gs": 1,

"gsn": "waiting_for_signal",

"wet": false,

"imp": false,

"t": 654321,

"time_my": "2026-05-30T14:32:01+08:00"

}

Optional `deviceName` may be added manually in Firebase for prettier app labels.

FS4. Receiver V1 vs V2 (prefer V2)

| Area | V1 (`RECEIVER_CONTROL_TOWER`) | V2 (`RECEIVER_CONTROL_TOWER_V2`) |
| --- | --- | --- |
| `time_my` in RTDB | NTP synced but not written | Written on every upload |
| RTDB write | Multiple `setInt`/`setString`/… calls | Single `updateNode` JSON |
| RTDB backoff | None | Dedicated exponential backoff |
| Hard recovery | None | After 6 consecutive failures |
| Token-not-ready | Counted as failure | Detected, 5 s wait, no failure increment |
| Post-`Firebase.begin()` delay | None | 7 s quiet period |
| Wi-Fi sleep | Auto-reconnect only | Also `WiFi.setSleep(false)` |

Both share: queue capacity 30, LoRa 433 MHz, same SPI map, flush up to 4/loop.

FS5. Mobile application (Flutter)

Source: `android-app/vest_tracker/lib/`

- Architecture (summary)

  - No Provider/Riverpod: config via `AppConfigScope` (`SharedPreferences`), Firebase via `FirebaseClientScope`.

  - `VestRepository`: streams `vest/` and parses device models (`vest_models.dart`).

  - Auth gate: signed-in `auth.uid` must equal `allowedUid` in app config.

  - Map: `flutter_map` + OpenStreetMap tiles; markers from RTDB lat/lon.

  - Notifications: `flutter_local_notifications`, channel `wet_alerts`.

  - Background: WorkManager task `vest_tracker_wet_monitor`, frequency from `pollIntervalMinutes` (clamped 15–240).

- Wet alert logic (app layer)

  - Foreground: `WetMonitorForeground` RTDB stream while app open.

  - Background: `WetMonitorBackground` periodic poll.

  - Notify only on `wet` false → true; first read per device sets baseline in prefs (`vest_tracker.lastWet.{deviceId}`).

  - Scope: all devices if `selectedDeviceId` empty, else single device.

Online heuristics (support note)

| Screen | Window | Use |
| --- | --- | --- |
| Live Map marker ripple | 5 minutes since last seen | “Online” animation |
| Devices card label | 10 minutes | Online/Offline text |

Last seen prefers `time_my` (Malaysia ISO) when present, otherwise falls back to other timestamp fields in model parsing.

Optional log upload

When `logsUploadEnabled`, `AppLogger` may push entries to RTDB path `appLogs/{clientInstanceId}/…`.

For screen-by-screen UI detail, (see MobileApp.pdf).

FS6. Normal runtime expectations

| Event | Typical timing |
| --- | --- |
| Scheduled LoRa TX | ~60 s per vest |
| Immediate LoRa TX | Debounced dry to wet |
| USB Serial JSON (vest) | ~10 s (when USB connected) |
| GPS cold start | Few minutes outdoors |
| Cloud visibility | Receiver on + Wi‑Fi + valid Firebase credentials |
| Background wet poll (app) | 15–240 min (user setting; OS may throttle) |

FS7. Reset behavior

- Vest power-cycle: GPS acquisition restarts. `lastGoodFix` cleared on boot, first LoRa may omit coordinates until new lock.

- Receiver power-cycle: Uploads pause until Wi‑Fi, NTP (for `time_my`), and Firebase ready; queued messages may flush afterward (subject to 30-message capacity).

FS8. Constants quick reference for transmitter

| Constant | Value |
| --- | --- |
| `VEST_ID` | `1` (per deployment flash) |
| `GPS_JSON_INTERVAL_MS` | `10000` |
| `LORA_TX_INTERVAL_MS` | `60000` |
| `GPS_NO_DATA_TIMEOUT_MS` | `2000` |
| Fix lock threshold | fix age < 2000 ms |
| `RAIN_DEBOUNCE_MS` | `80` |
| `RAIN_ANALOG_WET_THRESHOLD` | `2500` |
| `LORA_FREQ_HZ` | `433E6` |
| Serial / LoRa JSON buffers | `512` / `240` bytes |

FS9. Constants quick reference for receiver V2

| Constant | Value |
| --- | --- |
| `QUEUE_CAPACITY` | `30` |
| `MAX_FLUSH_PER_LOOP` | `4` |
| Backoff initial / max | `250` / `30000` ms |
| NTP timeout | `20000` ms |
| `POST_FIREBASE_BEGIN_QUIET_MS` | `7000` ms |
| Token-not-ready wait | `5000` ms |
| Max RTDB failures before hard recovery | `6` |
| Hard recovery cooldown | `60000` ms |
| `gsn` max length | `31` chars |
| `time_my` buffer | `32` chars |

FS10. Firmware updates (maintenance)

Firmware is updated by maintainers using Arduino IDE (or compatible tooling), the ESP32 board support package, and library versions referenced by each sketch. Operators should not reflash unless instructed. When reporting issues, include: device ID, symptom, approximate time, and any Serial Monitor capture at 115200 baud.

Data flow overview: (see SystemArchitecture.pdf). Fault finding: (see TroubleshootingGuide. pdf ).